AIS VRC-4  |  Frankfurt UAS  |  March 2026

VOICE RECOGNITION FOR ROBOT CONTROL

Team Handbook & Progress Report  —  Days 1–3

| KHUSHAL SINGH ASR & Preprocessing Lead macOS + Ubuntu 24 (VM) asr_module/ | RAKA SARKAR ROS 2 & Gazebo Lead Windows + Ubuntu 22.04 ros2_ws/ | TAIBAZ PATHAN Intent & State Machine Lead macOS + Ubuntu 24 (VM) intent_module/ |
| --- | --- | --- |

GitHub (private): github.com/khushal-singh/vrc-robot-control

# 1.  Project Overview

Build a robust, CPU-compatible voice recognition pipeline that integrates with ROS 2 Humble and controls a simulated robot (ROSWITHA) in Gazebo — validated entirely on pre-recorded WAV files, no cloud services, no microphone.

| Target Metric | Required Value |
| --- | --- |
| Clean audio recognition accuracy | > 85% |
| Heavy noise accuracy (SNR 0 dB) | > 70% (acceptable) |
| False activation rate | < 2% |
| End-to-end pipeline latency | < 5 seconds |
| Emergency stop execution | Immediate — bypass confirmation |
| Total evaluation samples | >= 100 samples |

# 2.  Repository Structure & Ownership

Each member owns their folder. Do NOT edit files in another member's folder without prior agreement.

vrc-robot-control/ │ ├── asr_module/ ← KHUSHAL (ASR engine, preprocessing, augmentation) │ ├── asr_engine.py ✅ COMPLETE │ ├── preprocessing/ │ │ ├── normalize.py ✅ COMPLETE │ │ └── vad.py ✅ COMPLETE │ └── augmentation/ │ └── snr_mixer.py ✅ COMPLETE │ ├── intent_module/ ← TAIBAZ (fuzzy matcher, state machine) │ ├── intent_mapper.py ⏳ In Progress │ ├── fuzzy_matcher.py ⏳ In Progress │ ├── vocabulary.py ⏳ In Progress │ ├── config.yaml ⏳ In Progress │ └── state_machine/ │ ├── states.py ⏳ In Progress │ ├── transitions.py ⏳ In Progress │ └── confirmation_logic.py ⏳ In Progress │ ├── ros2_ws/ ← RAKA (ROS 2 package, Gazebo integration) │ └── src/vrc_system/ │ ├── package.xml + setup.py + setup.cfg │ ├── config/commands.yaml + parameters.yaml │ ├── launch/asr_pipeline.launch.py │ ├── launch/gazebo_integration.launch.py │ └── vrc_system/ (audio_input_node, asr_node, intent_node, │ state_machine_node, command_node, cmd_vel_publisher) │ ├── audio_dataset/ ← SHARED (WAV files NOT pushed to GitHub) │ ├── clean/ ← 5 WAV files now, target 40 (8 speakers × 5 commands) │ ├── augmented/ ← 15 noisy files (5 × 3 SNR levels) │ ├── noise/background.wav │ └── metadata.csv ✅ pushed to GitHub (20 entries) │ ├── evaluation/ ← SHARED │ ├── batch_test_runner.py ✅ COMPLETE │ └── results/processed_metrics.csv │ ├── docs/diagrams/ ← SHARED (4 diagrams needed by Day 6) ├── scripts/generate_metadata.py ✅ COMPLETE └── README.md

# 3.  Setup Instructions

## 3.1  Clone the repo (everyone)

git clone https://github.com/khushal-singh/vrc-robot-control.git cd vrc-robot-control

## 3.2  Khushal & Taibaz — macOS

python3 -m venv vrc_env source vrc_env/bin/activate pip install faster-whisper soundfile numpy pandas rapidfuzz # Activate at the start of EVERY session: source ~/Downloads/vrc-robot-control/vrc_env/bin/activate

## 3.3  Raka — Ubuntu 22.04

python3 -m venv vrc_env source vrc_env/bin/activate pip install faster-whisper soundfile numpy pandas rapidfuzz # Activate at the start of EVERY session: source ~/vrc_env/bin/activate # ROS 2 Humble: follow docs.ros.org/en/humble/Installation.html separately # Build ROS 2 workspace: cd ros2_ws && colcon build && source install/setup.bash

## 3.4  Daily Git workflow

Always pull before starting, push when done. Work ONLY in your own folder.

git pull # always do this first # ... do your work ... git add asr_module/ # replace with your own folder git commit -m "Short description of what you did" git push

⚠️ RULES FOR ALL MEMBERS 1. Commit ONLY files inside your own assigned folder. 2. NEVER push WAV audio files — too large. Share via WhatsApp or Google Drive. 3. audio_dataset/metadata.csv IS pushed — update it after adding new samples. 4. If you break something, open a GitHub Issue immediately. 5. vrc_env/ is listed in .gitignore — it will never be pushed.

# 4.  Fixed Command Vocabulary

These are the ONLY valid commands. All three modules must use exactly these strings — no synonyms, no variants.

| # | Command String | Robot Action | Type |
| --- | --- | --- | --- |
| 1 | move forward | Positive linear.x velocity | Motion |
| 2 | move backward | Negative linear.x velocity | Motion |
| 3 | turn left | Positive angular.z velocity | Motion |
| 4 | turn right | Negative angular.z velocity | Motion |
| 5 | stop | Zero all velocities (conf >= 0.9 → bypass confirm) | Emergency |
| C1 | yes | Confirmation token — approve pending command | Confirm |
| C2 | no | Confirmation token — cancel pending command | Confirm |

# 5.  Agreed Technical Stack

| Component | Decision |
| --- | --- |
| ASR Engine | faster-whisper base model — CPU mode, int8 quantization |
| ROS Version | ROS 2 Humble — Ubuntu 22.04 ONLY (Raka's machine) |
| Python Version | 3.10+ (3.12.3 confirmed working on all machines) |
| Simulation | Gazebo — empty world baseline, ROSWITHA robot model |
| Audio Input | Pre-recorded WAV injection — NO live microphone |
| Fuzzy Matching | rapidfuzz library (Taibaz's intent_module) |
| Confidence Threshold | 0.75 default ¦ 0.9 for emergency stop bypass |
| Audio Format | 16 kHz mono WAV — all samples must match |
| Build Tool | colcon (ROS 2 workspace only) |
| JSON Output Schema | { "text": "move forward", "confidence": 0.87, "latency": 2.3 } |

# 6.  ROS 2 Topic Map  (Raka's Reference)

All five ROS 2 nodes communicate through these topics. This is the backbone of the pipeline.

| Topic | Msg Type | Publisher → Subscriber |
| --- | --- | --- |
| /audio/raw | Audio | audio_input_node → asr_node |
| /asr/text | String | asr_node → intent_node |
| /asr/intent | String | intent_node → state_machine_node |
| /voice/command | String | state_machine_node → command_node |
| /cmd_vel | Twist | command_node → Gazebo robot |

# 7.  Safety State Machine  (Taibaz's Reference)

All motion commands must pass through this 3-state safety machine before any velocity is published to /cmd_vel.

| State | Description | Exit Condition |
| --- | --- | --- |
| IDLE | Listening for voice commands. Robot stationary. | Valid command received (confidence >= 0.75) |
| WAITING_CONFIRMATION | Command detected. Waiting for YES or NO from user. | YES → EXECUTING ¦ NO or 5s timeout → IDLE |
| EXECUTING | Confirmed command published to /cmd_vel. | Command complete → return to IDLE |

🔴 Emergency Stop Override If 'stop' is detected with confidence >= 0.9, the state machine BYPASSES the WAITING_CONFIRMATION state entirely and publishes zero velocity immediately. ⏱️ Timeout — If no YES/NO is received within 5 seconds, the pending command is discarded and the machine returns to IDLE. No motion is executed.

# 8.  ASR Module — Khushal Singh  ✅  COMPLETE

## 8.1  Files delivered

| File | Purpose |
| --- | --- |
| asr_module/asr_engine.py | Core engine: WAV path in → JSON (text, confidence, latency) out |
| asr_module/preprocessing/normalize.py | 16 kHz resampling + stereo→mono + amplitude normalization |
| asr_module/preprocessing/vad.py | Energy-based VAD: 20 ms frames, threshold 0.01 RMS |
| asr_module/augmentation/snr_mixer.py | Mixes clean speech with noise at SNR 20 / 10 / 0 dB |
| evaluation/batch_test_runner.py | Batch accuracy tester across all augmented audio files |
| scripts/generate_metadata.py | Writes audio_dataset/metadata.csv (filename, command, SR, duration) |

## 8.2  asr_engine.py  —  full code  (Raka: use this inside asr_node.py)

import time, json, math, re from faster_whisper import WhisperModel model = WhisperModel("base", device="cpu", compute_type="int8") def transcribe(wav_path): start = time.time() segments, _ = model.transcribe(wav_path, language="en") segments = list(segments) latency = round(time.time() - start, 2) text = " ".join([s.text.strip() for s in segments]).lower() text = re.sub(r'[^\w\s]', '', text).strip() # strip punctuation confidence = 0.0 if segments: avg_logprob = sum([s.avg_logprob for s in segments]) / len(segments) confidence = round(min(1.0, math.exp(avg_logprob)), 3) return {"text": text, "confidence": confidence, "latency": latency} if __name__ == "__main__": import sys wav = sys.argv[1] if len(sys.argv) > 1 else "test.wav" print(json.dumps(transcribe(wav), indent=2))

## 8.3  How to call the ASR engine from Raka's asr_node.py

import sys sys.path.insert(0, '/path/to/vrc-robot-control') # adjust to your clone path from asr_module.asr_engine import transcribe result = transcribe('audio_dataset/clean/MoveForward.wav') # result = { 'text': 'move forward', 'confidence': 0.87, 'latency': 2.3 } if result['confidence'] >= 0.75: # publish result['text'] to /asr/text topic pass

## 8.4  Validation results (Day 3)

| Noise Condition | Correct / Total | Accuracy | Target |
| --- | --- | --- | --- |
| SNR 20 dB (light noise) | 5 / 5 | 100% | ✅ > 85% |
| SNR 10 dB (medium noise) | 5 / 5 | 100% | ✅ > 85% |
| SNR 0 dB (heavy noise) | 3 / 5 | 60% | ⚠️ Acceptable |
| Overall — augmented set | 13 / 15 | 86.7% | ✅ > 85% MET |

Key finding: All SNR 0 dB failures had confidence < 0.75 (range: 0.300–0.340), so they would be automatically rejected by Taibaz's state machine — no false command would ever be issued. All latencies were under 2.2 s, well within the 5 s target.

✅ Punctuation fix applied: Whisper adds trailing dots and exclamation marks. Fixed with: text = re.sub(r"[^\w\s]", "", text).strip() ✅ Re-recording fix: TurnRight.wav was re-recorded clearly. Result: TurnRight accuracy jumped from 0% to 100% across all SNR levels.

# 9.  Overall Project Status

| Deliverable | Owner | Status |
| --- | --- | --- |
| asr_engine.py + preprocessing + augmentation | Khushal | ✅ Complete |
| batch_test_runner.py + generate_metadata.py | Khushal | ✅ Complete |
| 86.7% accuracy on augmented audio confirmed | Khushal | ✅ Complete |
| 40 clean samples (8 speakers × 5 commands) | Khushal | ⏳ Pending |
| intent_mapper.py + fuzzy_matcher.py + vocabulary.py | Taibaz | ⏳ Pending |
| state_machine (states + transitions + confirmation) | Taibaz | ⏳ Pending |
| intent_module/config.yaml | Taibaz | ⏳ Pending |
| ROS 2 package vrc_system — all nodes | Raka | ⏳ Pending |
| Gazebo simulation loading ROSWITHA | Raka | ⏳ Pending |
| Both launch files working end-to-end | Raka | ⏳ Pending |
| Day 4 — Full pipeline integration on Raka's machine | All | ⏳ Pending |
| Day 5 — 100-sample evaluation suite | All | ⏳ Pending |
| Day 6 — 4 required diagrams + report sections | All | ⏳ Pending |
| Day 7 — Final report + slides + repo tag | All | ⏳ Pending |

# 10.  7-Day Sprint Plan

| Day | Khushal | Taibaz | Raka |
| --- | --- | --- | --- |
| Day 1 ✅ | Env setup — faster-whisper working on Ubuntu VM | Env setup — Python packages installed | Env setup — ROS 2 Humble installed |
| Day 2 ✅ | asr_engine.py + normalize.py + vad.py built | vocabulary.py + fuzzy_matcher.py | ROS 2 package vrc_system scaffolded |
| Day 3 ✅ | snr_mixer.py + batch_test_runner.py → 86.7% accuracy achieved | intent_mapper.py + state_machine/ files | All ROS 2 nodes + launch files |
| Day 4 | Support Raka's integration Collect more clean audio samples | Support Raka's integration | Full end-to-end pipeline integration on Ubuntu 22.04 |
| Day 5 | 100-sample batch test Accuracy + latency metrics | Accuracy tables Confusion matrix | End-to-end latency measurement + Gazebo demo |
| Day 6 | ASR Design + Evaluation sections Data flow diagram | Methodology + State Machine section State machine diagram | ROS 2 + Gazebo sections Node graph + architecture diagrams |
| Day 7 | Code review + docstrings + final push | Conclusion (1–2 pages) | README + Integration Guide + tag v1.0.0 |

VRC-4  |  AIS Frankfurt UAS  |  March 2026  |  Khushal Singh · Raka Sarkar · Taibaz Pathan